#### BTech Project

#### A Pentium Emulator

Peter McClymont

Department of Computer Science

University of Auckland

October 2000

Abstract

The Pentium processors are the most widely used processors around at the current time. They are popular for a number of reasons, including their power, flexibility, range of operating systems, software available and rapid advancement in speed and power. These are qualities that make it an attractive choice for the user. The power and flexibility of Pentium processors make them ideal for use in business and their reasonable price make them ideal for use in personal computers. Writing an emulator for the Pentium can be a very useful tool that can be used to run Pentium software on a range of different systems.

This project has extended an initial implementation of an emulator in which only the loader was completed. The final emulator produced as a result of this project will likely be used for future work to extend the emulator. The first task would be to extend the emulator to perform successfully on larger more complicated programs. This would then allow the extension of the emulator to perform a simulation of the Pentium processor.

Contents

Abstract		2

Acknowledgements	6

1 – Introduction	6

1.1 Current Status	7

1.2 Future Development	7

2 – Emulators	8

2.1 The Loader	9

2.2 The Decoder	9

2.3 Decoding the Instructions	10

2.4 Examples of Decoding	15

2.5 The Executor	17

3 - The Pentium Architecture 	18

3.1 Registers	18

3.2 Functional Description	22

3.3 Branch Target Prediction	23

3.3.1 Correct Predictions	25

3.3.2 Incorrect Predictions	25

3.4 Pentium Data Organisation	25

3.5 Pentium Addressing Modes	26

3.6 Instruction Format	28

3.7 Pentium Instruction Set	30

4 - The ELF File Format	32

4.1 File Format	33

4.2 ELF Library	34

5 - Program Structure	36

5.1 System Calls	40

6 - Results		43

7 - Problems	46

8 - Conclusion	48

A - Source Code	49

Bibliography	50

List Of Figures

2.1 - Information Collected For Instructions				10

2.2 - The 256 possibilities for the ModR/M byte				12

2.3 - Effective address calculation using the SIB byte			13

2.4 - The 256 possibilities for the SIB byte					14

3.1 - The Pentium Registers						21

3.2 - The Pentium Processor Layout					22

3.3 - The U and V Pipelines						23

3.4 - Branch Target Prediction Process					24

3.5 - Storage of 0x80503A21, a 32 bit piece of data in memory		26

3.6 - Storage of 0x459a, a 16 bit piece of data in memory			26

3.7 - Pentium Instruction Format						28

4.1 - The linking and execution view of the ELF file format		33

4.2 -  Information extracted by the ELF library				34

5.1 - Program Structure							37

5.2 - The Sections Stored in the core					38

# Acknowledgments

##### I would like to thank my supervisor, Dr S. Manoharan who has supported me over the course of this project. With his help I have been able to learn a great dealt about the Linux operating system, about the Pentium Architecture, about the C++ language and about emulators in general.

##### Chapter 1

##### Introduction

An Emulator is a program that takes software written for a different architecture and runs it on the current system. The aim of the emulator is to run the program to the original specifications. For performance reasons a lot of sacrifices are made in terms of how closely it imitates the original architecture.

Am Emulator is suitable if we just simply want to run software that is written for a different system, but if we wish to do performance evaluations and test a specific system, or program, then an emulator is not suitable for this task. For example, if our aim is to run an old arcade game, such as PACMAN on a P.C. then an emulator will be fine in managing this task. If we wish to design a new processor, and evaluate the design before starting production, then an emulator will not be sufficient to mimic the architectural design, and will not be suitable for this task.

The goal of this project is to design and implement an emulator for the Pentium processor, with the view that it can be later extended to a simulator.

This report describes the design aspects and implementation details of the Pentium emulator, called Durvasa. The emulator is written in C++, and runs under the Linux operating system.

1.1 Current Status

Currently durvasa can perform basic instruction emulations. Getting even to this stage has not been easy. The ground work has been done, and this opens the door for someone else to extend this emulator in the future.

Currently durvasa has a complete working loader. This part of the code doesn’t really need to be changed in the future when more complicated emulations are carried out as the loading process will not change. The decoder is also complete, and would only require basic modifications for the addition of new instructions as more complicated programs are emulated. The executer is the most difficult part of an emulator to implement. Currently a basic executor has been implemented that will perform on simple programs, but with more work in the future it can be extended to perform more complicated emulations.

1.2 Future Development

It is likely that this project will be handed on to someone else next year to extend and improve the project. The program currently performs simple emulations, but with a bit of work the program will be able to perform emulations on larger programs, and the program could be modified to run a simulation of the Pentium Architecture. This would involve producing a program which more closely runs like the Pentium processor. Things that this could involve are the addition of instruction pipelining, a caching system, and also a memory structure that more closely represents the Pentium.

### Chapter 2

Emulators

An emulator is a piece of software that allows the user to run software written for a specific system on a different system. An example of this is a MAC emulator written for a PC. This would allow software written for a MAC to run on a PC as if it was actually a MAC. This is an example of an emulator for a serious style of application, but this is not the only type of emulator. A current trend is to write and use gaming emulators. These allow the users to play old style arcade or computer games on the current range of computers available today. Emulators allow users the flexibility of running software on a current machine that may not be available for that particular machine, without having to go out and buy, hire or borrow a new system.

An emulator usually contains three parts, the loader, which loads the file into the created memory structure, the decoder which goes through the text segment (where the instructions are located) extracting out instructions and setting up a data structure for each one, and the executor which takes the information from the data structure and runs the specific instruction as required.

2. 1 The Loader

### The loader has the task of setting up the code that is being emulated into the specific memory structure. The first thing that must be done in achieving this is to load the header and to find the required information about the file. This will allow us to load the separate sections of the file into the memory. Linux uses the ELF file format to decode object files, and the ELF library (libelf) is used in durvasa to allow us to load the file and understand the different sections. See chapter 5 for more information on the ELF file format and the ELF library.

2.2 The Decoder

The decoder starts at the address in the EIP (The instruction pointer register), and analyses the bytes following that position in the memory. It is given the job of extracting instructions and the corresponding data to go along with the instructions, simply by looking at a stream of bytes. It decides whether instruction prefixes are present, how much opcode information is present, whether a ModR/M and/or SIB byte is present and whether immediate information and/or relative information is present. The decoder puts all the information that it collects about the current instruction in a data structure. The data structure in durvasa holds information as shown below.

| Parameter | What the Parameter Represents |
| --- | --- |
| int noprefix | Tells us how many prefixes have been found. |
| int prefixes[4] | Holds the found prefixes. This can be a maximum of 4 |
| int opcode | Hold the main opcode value for the instruction |
| int opcode2 | Hold the second opcode value for the instruction, if neccesary |
| int modrm | Holds the Mod/RM byte, if neccesary |
| int SIB | Holds the SIB byte, if neccesary |
| int relativeLength | Tells us how many bytes of relative information has been found. |
| int relative[4] | Holds the relative information |
| int immediateLength | Tells us how many bytes of immediate information has been found. |
| int immediate[4] | Holds the immediate information |

Figure 2.1 – Information Collected For Instructions

2.3 Decoding The Instructions

Decoding the instructions is not a very easy process because of the nature of the Pentium instruction set. In order to decide which parts of the instruction format are needed we have to read the opcode, and modr/m (if present) bytes and understand which addressing mode is being used. Following is described the proceedure for Decoding with respect to the separate instruction format.

The instruction prefixes. The decoder looks at the first byte being pointed to by the EIP and decides whether it is an instruction prefix. If it is then it stores this information in the first two fields of the data structure. We can have up to 4 prefixes, so if a prefix is found the process continues until a non-prefix byte is reached.

The Opcode. The next byte the decoder will be looking at will be an opcode value. The opcode value tells us a number of things.

- What instruction follows.

- Whether extra opcode information is present in another opcode byte following the first.

- Whether extra opcode information is present in the 3-bit reg/opcode field in the ModR/M byte. This will be the case for instructions that only have one operand.

- Whether immediate information is present (this can also be specified by the ModR/M byte).

- Whether relative information is present (this again can be specified by the ModR/M byte).

The ModRM Byte. One of the most important things that the decoder does, is decodes the ModRM byte. The ModRM byte is used in any instruction specifying an r/m32 (32 bits from either one of the 24 addressing modes or one of the 8 registers), an r/m16, or an r/m8 as an operand. The ModRM tells us whether the operand is one of the 24 addressing modes or one of the 8 registers. The ModR/M also can specify a second operand which will be one of the 8 registers, or this can be extra opcode information if a second operand is not required by that particular instruction No instruction can have more than one r/m operand.

Out of the 256 values that the ModR/M can take, 132 of these require relative information, which will be present after the ModR/M or SIB byte, and 24 of these require an SIB byte, which will follow after the ModR/M byte. Bellow shows a table that specifies the 255 values of the ModR/M byte, what operands these represent, and what extra information will be present after the ModR/M.

| r8(/r) |  |  | AL | CL | DL | BL | AH | CH | DH | BH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r16(/r) |  |  | AX | CX | DX | BX | SP | BP 1 | SI | DI |
| r32(/r) |  |  | EAX | ECX | EDX | EBX | ESP | EBP | ESI | EDI |
| /digit (Opcode) |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| REG = |  |  | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| Effective address | Mod | Rm | Value of ModR/M Byte (in Hexadecimal) |  |  |  |  |  |  |  |
| [EAX] | 00 | 000 | 00 | 08 | 10 | 18 | 20 | 28 | 30 | 38 |
| [ECX] |  | 001 | 01 | 09 | 11 | 19 | 21 | 29 | 31 | 39 |
| [EDX] |  | 010 | 02 | 0A | 12 | 1A | 22 | 2A | 32 | 3A |
| [EBX] |  | 011 | 03 | 0B | 13 | 1B | 23 | 2B | 33 | 3B |
| [--][--] |  | 100 | 04 | 0C | 14 | 1C | 24 | 2C | 34 | 3C |
| Disp32 |  | 101 | 05 | 0D | 15 | 1D | 25 | 2D | 35 | 3D |
| [ESI] |  | 110 | 06 | 0E | 16 | 1E | 26 | 2E | 36 | 3E |
| [EDI] |  | 111 | 07 | 0F | 17 | 1F | 27 | 2F | 37 | 3F |
| Disp8[EAX] | 01 | 000 | 40 | 48 | 50 | 58 | 60 | 68 | 70 | 78 |
| Disp8[ECX] |  | 001 | 41 | 49 | 51 | 59 | 61 | 69 | 71 | 79 |
| Disp8[EDX] |  | 010 | 42 | 4A | 52 | 5A | 62 | 6A | 72 | 7A |
| Disp8[EBX] |  | 011 | 43 | 4B | 53 | 5B | 63 | 6B | 73 | 7B |
| Disp8[--][--] |  | 100 | 44 | 4C | 54 | 5C | 64 | 6C | 74 | 7C |
| Disp8[EBP] |  | 101 | 45 | 4D | 55 | 5D | 65 | 6D | 75 | 7D |
| Disp8[ESI] |  | 110 | 46 | 4E | 56 | 5E | 66 | 6E | 76 | 7E |
| Disp8[EDI] |  | 111 | 47 | 4F | 57 | 5F | 67 | 6F | 77 | 7F |
| Disp32[EAX] | 10 | 000 | 80 | 88 | 90 | 98 | A0 | A8 | B0 | B8 |
| Disp32[ECX] |  | 001 | 81 | 89 | 91 | 99 | A1 | A9 | B1 | B9 |
| Disp32[EDX] |  | 010 | 82 | 8A | 92 | 9A | A2 | AA | B2 | BA |
| Disp32[EBX] |  | 011 | 83 | 8B | 93 | 9B | A3 | AB | B3 | BB |
| Disp32[--][--] |  | 100 | 84 | 8C | 94 | 9C | A4 | AC | B4 | BC |
| Disp32[EBP] |  | 101 | 85 | 8D | 95 | 9D | A5 | AD | B5 | BD |
| Disp32[ESI] |  | 110 | 86 | 8E | 96 | 9E | A6 | AE | B6 | BE |
| Disp32[EDI] |  | 111 | 87 | 8F | 97 | 9F | A7 | AF | B7 | BF |
| EAX/AX/AL | 11 | 000 | C0 | C8 | D0 | D8 | E0 | E8 | F0 | F8 |
| ECX/CX/CL |  | 001 | C1 | C9 | D1 | D9 | E1 | E9 | F1 | F9 |
| EDX/DX/DL |  | 010 | C2 | CA | D2 | DA | E2 | EA | F2 | FA |
| EBX/BX/BL |  | 011 | C3 | CB | D3 | DB | E3 | EB | F3 | FB |
| ESP/SP/AH |  | 100 | C4 | CC | D4 | DC | E4 | EC | F4 | FC |
| EBP/BP/CH |  | 101 | C5 | CD | D5 | DD | E5 | ED | F5 | FD |
| ESI/SI/DH |  | 110 | C6 | CE | D6 | DE | E6 | EE | F6 | FE |
| EDI/DI/BH |  | 111 | C7 | CF | D7 | DF | E7 | EF | F7 | FF |

Figure 2.2 – The 256 possibilities for the ModR/M byte

The SIB byte. The SIB byte is required by 3 of the addressing modes specified by the ModR/M byte. This accounts for 24 of the 256 values that the ModR/M byte can take.

The SIB byte allows us to make an address calculation that can be made up of the following components: -

• Base - The value of one of the general-purpose registers.

• Index - The value of one of the general-purpose registers.

• Scale factor—A value of 1, 2, 4, or 8 that is multiplied by the index value.

The offset that results from adding these components is called an effective address. Each of these components can have either a positive or negative (2s complement) value, with the exception of the scaling factor. The calculation is made as follows.

Offset = Base + (Index * Scale) + Displacement

The SIB byte is useful for accessing data stored in a structure, for example an array of 4 byte elements. The base will be set to point to the start of the data, the index will specify which element we wish to access, and the scale factor will be set to the size of each element in the array. Below is shown the combinations of registers that can be used for calculating the SIB byte, and furrther below is shown a table of the 256 values of the SIB byte.

Base	Index	Scale	Displacement

EAXEBXECXEDXESPEBPESI

Figure 2.3 – Effective address calculation using the SIB byte.

| R32(/r) |  |  | EAX | ECX | EDX | EBX | ESP | EBP | ESI | EDI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base = |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Base = |  |  | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| Scaled index | SS | Index | Value of SIB Byte (in Hexadecimal) |  |  |  |  |  |  |  |
| [EAX] | 00 | 000 | 00 | 08 | 10 | 18 | 20 | 28 | 30 | 38 |
| [ECX] |  | 001 | 01 | 09 | 11 | 19 | 21 | 29 | 31 | 39 |
| [EDX] |  | 010 | 02 | 0A | 12 | 1A | 22 | 2A | 32 | 3A |
| [EBX] |  | 011 | 03 | 0B | 13 | 1B | 23 | 2B | 33 | 3B |
| none |  | 100 | 04 | 0C | 14 | 1C | 24 | 2C | 34 | 3C |
| [EBP] |  | 101 | 05 | 0D | 15 | 1D | 25 | 2D | 35 | 3D |
| [ESI] |  | 110 | 06 | 0E | 16 | 1E | 26 | 2E | 36 | 3E |
| [EDI] |  | 111 | 07 | 0F | 17 | 1F | 27 | 2F | 37 | 3F |
| [EAX*2] | 01 | 000 | 40 | 48 | 50 | 58 | 60 | 68 | 70 | 78 |
| [ECX*2] |  | 001 | 41 | 49 | 51 | 59 | 61 | 69 | 71 | 79 |
| [EDX*2] |  | 010 | 42 | 4A | 52 | 5A | 62 | 6A | 72 | 7A |
| [EBX*2] |  | 011 | 43 | 4B | 53 | 5B | 63 | 6B | 73 | 7B |
| none |  | 100 | 44 | 4C | 54 | 5C | 64 | 6C | 74 | 7C |
| [EBP*2] |  | 101 | 45 | 4D | 55 | 5D | 65 | 6D | 75 | 7D |
| [ESI*2] |  | 110 | 46 | 4E | 56 | 5E | 66 | 6E | 76 | 7E |
| [EDI*2] |  | 111 | 47 | 4F | 57 | 5F | 67 | 6F | 77 | 7F |
| [EAX*4] | 10 | 000 | 80 | 88 | 90 | 98 | A0 | A8 | B0 | B8 |
| [ECX*4] |  | 001 | 81 | 89 | 91 | 99 | A1 | A9 | B1 | B9 |
| [EDX*4] |  | 010 | 82 | 8A | 92 | 9A | A2 | AA | B2 | BA |
| [EBX*4] |  | 011 | 83 | 8B | 93 | 9B | A3 | AB | B3 | BB |
| none |  | 100 | 84 | 8C | 94 | 9C | A4 | AC | B4 | BC |
| [EBP*4] |  | 101 | 85 | 8D | 95 | 9D | A5 | AD | B5 | BD |
| [ESI*4] |  | 110 | 86 | 8E | 96 | 9E | A6 | AE | B6 | BE |
| [EDI*4] |  | 111 | 87 | 8F | 97 | 9F | A7 | AF | B7 | BF |
| [EAX*8] | 11 | 000 | C0 | C8 | D0 | D8 | E0 | E8 | F0 | F8 |
| [ECX*8] |  | 001 | C1 | C9 | D1 | D9 | E1 | E9 | F1 | F9 |
| [EDX*8] |  | 010 | C2 | CA | D2 | DA | E2 | EA | F2 | FA |
| [EBX*8] |  | 011 | C3 | CB | D3 | DB | E3 | EB | F3 | FB |
| none |  | 100 | C4 | CC | D4 | DC | E4 | EC | F4 | FC |
| [EBP*8] |  | 101 | C5 | CD | D5 | DD | E5 | ED | F5 | FD |
| [ESI*8] |  | 110 | C6 | CE | D6 | DE | E6 | EE | F6 | FE |
| [EDI*8] |  | 111 | C7 | CF | D7 | DF | E7 | EF | F7 | FF |

Figure 2.4 – The 256 possibilities for the SIB byte.

The Immediate byte(s). The Immediate byte(s) are required by some of the addressing modes, for example the direct addressing and immediate addressing modes. The decoder will know whether immediate bytes are present, and how many are present by looking at the specific instruction being decoded and also by looking at the ModR/M byte.

The Relative bytes(s). The Relative byte(s) are required by some of the addressing modes, for example the indexed addressing and based addressing modes. The decoder will know whether Relative bytes are present, and how many are present by again looking at the specific instruction being decoded and also by looking at the ModR/M byte.

2.4 Examples Of Decoding

8b 4d 08	MOVL   0x8[EBP], ECX

The steps taken to decode the above are outlined below:-

1) The decoder looks for prefix values, and finds none

2) The decoder finds an opcode value of 0x8b and sets the opcode field of the data structure to 0x8b. It recognizes this instruction as movl r/m32, r32

3) The decoder knows that for this instruction with am r/m32 operand, we need a ModR/M. We get the value 0x4d, and place this in the modrm field of the data structure.

4) We look at the modrm byte, which tells us that the addressing mode to use for the r/m32 field is disp8 [EBP] (32 bits of information from the address specified by EBP + an 8 bit displacement), and that the r32 operand for this instruction is ECX.

5) We know that for the disp8 [EBP] operand we have a disp8 (one byte of relative information) so we read the next byte, which specifies this – 0x08, and place this in the relative field in the data structure and update the relativeLength field.

66 89 ca	MOVW   CX, DX

The steps taken to decode the above are outlined below:-

1) The decoder looks for prefix values, and finds one – 0x66. It adds this to the prefix field of the data sructure and updates the noprefix field. It looks for more prefix values and doesn’t find any.

2) The decoder finds an opcode value of 0x89 and sets the opcode field of the data structure to 0x89. It recognizes this instruction as MOVL r32, r/32.

3) The decoder knows that for this instruction with an r/m32 operand, we need a ModR/M. We get the value 0xca, and place this in the modrm field of the data structure.

4) We look at the modrm byte, which tells us that the addressing mode to use for the r/m32 operand is simply the register EDX and that the r32 operand for this instruction is ECX, but the prefix of 0x66 is an operand size override from 32 bits to 16 bits, making the r/m32 operand DX and the r32 operand CX.

80 fa 7f	CMPB   0x7f, DL

The steps taken to decode the above are outlined below.

1) The decoder looks for prefix values, and finds none

2) The decoder finds an opcode value of 0x80 and sets the opcode field of the data structure to 0x80. It recognizes this instruction as one which requires the opcode field of the ModR/M. The opcode field of the ModR/M is the bits 3, 4, and 5 of the ModR/M field. These bits are extracted and the instruction CMP imm8, r/m8 is decided upon.

3) The decoder knows that for this instruction with an r/m8 operand, we need a ModR/M. We get the value 0xfa, and place this in the modrm field of the data structure.

4) We look at the modrm byte, which tells us that the addressing mode to use for the r/m8 field is one of the 8 registers, dl.

5) We know that for the imm8 operand we need one byte of immediate information so we read the next byte, which specifies this – 0x7f, and place this in the immediate field in the data structure and update the immediate Length field.

2.5 The Executor

The executor takes the data that is received from the decoder and executes the appropriate instructions. The executor is the most difficult part of the emulator to implement and debug. The execution of the instructions is not often the problem, the problem is what needs to be done before and after the implementation. Firstly we need to get the operands for the instruction, which is a more involved process than in the decoder, also we need to set the flag register after the instruction is implemented. If either of these things are done incorrectly, then the whole emulation of the program will be affected, and the emulation will be likely to fail.

The executor sets the flag register depending on the results of certain arithmetic and logic instructions. Certain bits of the flag register represent specific conditions, the most important being the carry flag (CF), overflow flag (OF), sign flag (SF), parity flag (PF), zero flag (ZF), and the adjust flag (AF). These flags are used to change the flow of execution in the program. This allows the assembly programmer, or the compiler to make decisions and jump to specific parts of the program according to the results.

The executor takes care of the registers, and also performs the system calls as necessary. These components make for a system which is similar to the Pentium Architecture, but in a simpler way. In the future if an attempt to implement a simulator is undertaken then this structure will have to be implemented more closely to the original Pentium system.

### Chapter 3

#### The Pentium Architecture

Pentiums are the newest and fastest processors released by Intel. They quickly took the lead in the processor market. These processors are used for everything imaginable from home and business computing systems, to imbedded applications.

The Intel range of processors has included the 8088, 8086, 80186, 80286, 80386, 80486 and now Pentiums. Intel has developed Pentiums to be able to run programs from any of the 80x86 line, although such a program will run much faster on Pentium processors.

3.1 Registers

The Pentium has included four 32 bit data registers known as the accumulator (EAX), base (EBX), count (ECX), and data (EDX) registers. The lower 16 bits of these registers can be used separately as the AX, BX, CX, and DX registers. These four 16 bit registers can be used as eight separate 8-bit registers. We can achieve this by referring to the higher and lower halves of these registers, AH refers to the lower 8 bits of the register and AL refers to the higher 8 bits in the AX register. The main reason that these registers can be accessed in this way is to allow backward compatibility with older Intel Processors. A number of the instructions in the Pentium instruction set still refer to 8 bit and 16 bit operands for this reason.

Five other 32 bit pointer registers are available. These are the stack pointer (ESP), base pointer (EBP), source index (ESI), destination index (EDI), and the instruction pointer (EIP). The lower sixteen bits of all but the EIP can be addressed separately as SP, BP, SI, DI

The general-purpose registers are the EAX, EBX, ECX, EDX, ESP, EBP, ESI, EDI, and EIP.

The Pentium has six 16 bit segment registers which control all I/O and memory operations. The code segment (CS) is used for instruction fetches, data segment (DS) is used when reading and writing data, stack segment (SS) is used for stack operations such as calling and returning from subroutines, extra segment (ES) is an extra register that can be used however the programmer desires. In the 80386 two extra segment registers FS and GS were added. These again can be used for anything.

A 32 bit flag register EFLAGS is also made available to hold results on arithmetic and logic instructions. The flags contain sign, zero, carry and parity flags and also other flags that are used only in protected mode. Below an outline of the status flags is given. [INTEL 99a]

Overflow flag (OF). Set if the integer result is too large a positive number or too small a negative number (excluding the sign-bit) to fit in the destination operand; cleared otherwise. This flag indicates an overflow condition for signed-integer (two’s complement) arithmetic.

Carry flag (CF). Set if an arithmetic operation generates a carry or a borrow out of the most significant bit of the result; cleared otherwise. This flag indicates an overflow condition for unsigned-integer arithmetic. It is also used

in multiple-precision arithmetic.

PF (bit 2) Parity flag. Set if the least-significant byte of the result contains an even

number of 1 bits; cleared otherwise.

AF (bit 4) Adjust flag. Set if an arithmetic operation generates a carry or a borrow from bit 3 of the result; cleared otherwise. This flag is used in binary-coded decimal (BCD) arithmetic.

ZF (bit 6) Zero flag. Set if the result is zero; cleared otherwise.

SF (bit 7) Sign flag. Set equal to the most-significant bit of the result, which is the sign bit of a signed integer. (0 indicates a positive value and 1 indicates a negative value.)

31		      General Purpose Registers		0

EAX

EBX

ECX		EDX

ESP

EBP

EDI

ESI

16         Segment Registers	0

CS

DS

SS

ES

FS

GS

31                           Status and Control Registers	0

EIP

EFLAGS

Figure 3.1: The Pentium Registers

3.2 Functional Description

The diagram below shows the layout of the Pentium processor.

Branch prediction Logic

Clock Driver

Code Cache

Instruction Fetch

Code TLB

Instruction Decode

Complex Instruction Support

Bus Interface Logic

Data

Cache

Data TLB

Pipelined Floating Point

Superscalar Integer Execution Units

MP Logic

Figure 3.2: The Pentium Processor Layout [DIGITAL 00]

The Pentium processor's superscalar architecture is implemented using two integer pipelines, commonly referred to as the U and V pipes. The two pipelines differ from each other only in that the U-pipe can execute the entire instruction set, whereas the V-pipe is only able to execute "simple" instructions. This is because the V-pipeline does not have a barrel shifter (A barrel shifter can shift the bits in a word by as many places as needed in a single operation) as part of its Arithmetic Logic Unit (ALU). The most optimum performance occurs when two instructions are being executed in parallel (one in each pipe). The more often this can occur, the better.

Each pipeline consists of five stages as denoted by the block diagram. These stages are Prefetch, Decode 1, Decode 2, Execute, and Writeback.

Prefetcher

V Pipe

Decode 1

Decode 2

Decode 1

Decode 2

Execute

FP Execute 1

Writeback

Execute

U Pipe

FPU

Figure 3.3: The U and V Pipelines [Digital 00]

3.3 Branch Target Prediction

One of the inherent issues that occur when instructions are prefetched and queued in a prefetch buffer is how to handle conditional branch instructions. Prefetchers in older processors assumed that the execution unit would want the next instruction from the next sequential memory location.

The answer to this dilemma was to add Branch Prediction. Branch Prediction logic permits the Pentium processor to make more intelligent decisions regarding what information to prefetch from memory. The Branch Prediction process is shown in Figure 3.4.

U Pipe

Code Cache

Prefetcher

Queue

A

Queue

B

Decode 1

Decode 1

History

Branch Target Address

Branch Instruction Address

V Pipe

Branch Target Buffer

(BTB)

32 Bits

32 Bits

Figure 3.4: Branch Target Prediction Process [Digital 00]

When a branch instruction is fetched, the prediction logic predicts whether it will be taken or not (it does this by examining the execution history of the instruction). If the branch is not taken, it continues fetching instructions from the next sequential memory location of the current queue. If it predicts the branch will be taken, the prefetcher will switch to the other queue and begin fetching instructions from memory, starting at the branch target address

The prediction is accomplished using the Branch Target Buffer (BTB). The BTB is capable of storing the information for up to 256 unique branch instructions. The BTB contains the following information:

- Branch Instruction Address

- Branch Target Address

- History Bits

3.3.1 Correct Predictions

When predictions are correct, pipeline stalls are prevented. The prefetcher fetches code from the correct address (either the Branch Target Address if the branch is taken, or the next sequential address after the branch instruction address if not taken) and begins decoding the instructions. The prediction is verified in the "execute stage."

3.3.2 Incorrect Predictions

If the prediction is incorrect, the pipeline is flushed and a stall occurs while the pipe is refilled. A three-clock penalty occurs for any branch instruction mispredicted in the U-pipe, and for conditional branches mispredicted in the V-pipe. A four-clock penalty occurs for unconditional branches mispredicted in the V-pipe

Most importantly, this feature is completely software transparent, and current existing code benefits from the performance gain. [Digital 00]

3.4 Pentium Data Organisation

Pentium’s have instructions that manipulate 1, 8, 16 or 32 bits of data at a time. Commonly 8 bits are referred to as a byte, 16 bits are referred to as a word and 32 bits are known as a double word or a long word.

Compared to most other processors, Pentium’s are quite unique in the way that data is stored in memory. If a 16 bit number is stored in the 8 bit wide memory, it is stored in a way different to what we would expect. The higher 8 bits of the word is stored in the first memory space and the lower 8 bits of the word is stored in the second memory space. This is known as byte swapping. Below is shown the way that 16, and 32 bits of data are organized in memory.

Address	    Address + 1	 Address + 2	Address + 3

21	 3A	50	 80

Figure 3.5 – Storage of 0x80503A21, a 32 bit data in memory

Address	    Address + 1

9a	45

Figure 3.6 – Storage of 0x459a, a 16 bit data in memory

3.5 Pentium Addressing Modes

The Pentium has a wide variety of addressing modes, which are used in different circumstances.

The simplest is immediate addressing. The information is included directly in the instruction. An example of this is:

MOV ECX, 0x045E0E3F

The value that is put into the CX register is directly included within the instruction.

Moving the data between registers is referred to as register addressing. This is very fast, as it is not required to get information from the memory to do such an instruction (except for the instruction fetch). An example is:

ADD ECX, EDX

This instruction adds the ECX and EDX registers together and puts the result into the ECX register. No extra information is needed from memory after the initial instruction fetch so the execution time for this instruction will be fast.

Direct addressing is when we get data from a specific address. An example of direct addressing is:

MOV EDX, [0x6050578A]

This fetches the data from the address 0x6050578A and places it in the EDX register.

When a register is placed in the square brackets this is referred to as register indirect addressing. An example of register indirect addressing is

MOV EAX, [EDX]

This will not take EDX as a literal value but will look at the memory location that EDX points to and then copy that data over to the register EAX.

A slight addition to the above gives us indexed addressing. An example of this follows:

MOV EAX, 0x15[EDX]

This takes the value at the address (EDX + 0x15) and puts it into the EAX register.

When the register used is the base pointer (EBP) we refer to this as based addressing. An example of this is:

MOV EAX, 0x9[EBP]

This is useful with large tables or arrays of data, or for getting parameters when making subroutine calls. The SIB byte can also be used for accessing large tables or arrays.

The information about which addressing mode the instruction uses is defined in the Mod R/M byte of the instruction. There are a total of 24 different addressing modes that can be used.

3.6 Instruction Format

All Pentium instructions are based on the format shown in the diagram. They consist of up to four instruction prefixes, a one or two byte opcode, an addressing form specifier (if required) – including a Mod R/M byte and maybe an SIB byte, a displacement field (if required) and an immediate field (if required).

Instruction Prefixes

Opcode

ModR/M

SIB

Displacement

Immediate

UP to four prefixes of 1-byte each (optional)

1 or 2 byte opcode

1 byte

(if required)

Address displacement of 1, 2, or 4 bytes or none

Immediate data of

1, 2, or 4 bytes or none

1 byte

(if required)

6 5

6 5

7

7

3 2

3 2

7

0

Mod

Reg/

Opcode

R/M

Scale

Index

Base

Figure 3.7: Pentium Instruction Format

Instruction Prefixes. The instruction prefixes fit into four categories, and one from each category can be used for any particular instruction if required. These four categories are as follows

- Lock and repeat

- Segment override

- Operand size override

- Address size override

Opcode. The opcode value can be either one or two bytes long. It is possible for extra opcode information to be present in the Mod R/M byte in bits 3-5.

Mod R/M Byte. The addressing form specifier byte (called the Mod R/M) is present for most instructions that refer to an operand in memory. The Mod R/M byte has three fields of information

- The mod field combines with the R/M field to specify 32 values, 24 addressing modes and 8 register values.

- The reg/opcode field either has a register number or 3 more bits of opcode

information. Which of the two it will be is decided by which instruction we have.

- The r/m field can specify a register or a memory operand. It is combined with the mod field to represent an addressing mode.

SIB Byte. In certain circumstances an SIB byte has to be used along with the Mod R/M byte to specify the addressing form. The SIB byte is useful when accessing large tables or arrays of data. The SIB byte has three fields of information.

- The scale field gives us the scale factor.

- The index field gives us the register number of the index register.

- The base field gives us the register number of the base register.

Displacement Bytes. Some addressing forms require displacement information. This can be 0, 1, 2 or 4 bytes. This follows directly after the Mod R/M or SIB bytes.

Immediate Bytes. Some addressing forms require immediate information. This also can be 0, 1, 2 or 4 bytes. This follows directly after the displacement information. [Intel 99b]

3.7 Pentium Instruction Set

The Pentium instruction set has a large number of instructions with many of them catering for backward compatibility. Below is a summary of some of the basic instructions and what they do.

Arithmetic Operations

ADD 	– Adds two values

SUB	- Subtracts one operand from the other

DEC 	– Decrements a value by 1

DIV	- Divides two values

MUL	- Multiplies two values

INC	- Increments a value by 1

Comparison Operations

CMP	- Compares two numbers by subtracting the numbers

TEST	- Compares two numbers by performing an AND operation

Branch Operations

CALL	- Calls a subroutine. Saves the return address on the stack

JMP	- Jumps to an address. Doesn’t save the return address

JCON	- Jumps on a certain condition. Usually follows a CMP or TEST.

RET	- Return from a subroutine. Pops the return address from the stack

Logic Operations

NEG	- Performs a twos compliment negation on the operand

NOT	- Reverses all of the bits in the operand.

OR	- Performs a logical OR on the operand

XOR	- Performs a logical XOR on the operand

AND	- Performs a logical AND on the operand

Other Important Operations

INT	- Performs an interrupt. An INT 0x80 runs a system call.

LEA	- Loads an effective address

MOV	- Moves one operand to another

PUSH	- Push an operand onto the stack

POP	- Pops an operand onto the stack

XCHG	- Exchanges register values

Chapter 4

The ELF File Format

The Executable and Linking Format is a file format that describes executables in many different operating systems. The standard was developed  by UNIX as  part of the Application binary interface. The ELF standard has the intention of streamlining software development by providing binary interface definitions that can be used in multiple operating environments. This is intended to reduce the need to recode or recompile code across different systems.

The ELF format is only used for files with native code. This is code that is intended to be run directly by the processor of the machine.

The Elf file format has three different types of object files.

i) A relocatable file – this holds code suitable to be linked with other code to produce an executable file.

ii) An executable file - this holds a program suitable to be loaded and executed.

iii) A shared object file – this contains code that is suitable to be linked in two different contexts. It can be linked dynamically to produce a process image or it can be linked by the link editor.

# 4.1 File Format

The ELF file format applies both to execution and linking of the code. The diagram below shows the linking view of the ELF format on the left and the execution view of the ELF format on the right.

Linking View

ELF Header

Program header table

Optional

Section 1

…

Section n

…

…

Section Header Table

Execution View

ELF Header

Program header table

Optional

Segment 1

Segment 2

…

Section Header Table

Figure 4.1: The linking and execution view of the ELF file format

The ELF header contains information about the object file. This tells us about the organisation of the file and allows the operating system, or the emulator in our case to load it and successfully create a process image.

The Program header table, if present, tells the system how to create a process. Relocatable files do not need a program header, but files that are used to create a process image do.

The Sections contain the instructions, relocation information, data and the symbol table.

The Section header table has information about the sections in the file. Each section has an entry in the table. This holds information such as the section name and the section size.  [ELF 99]

4.2 The ELF Library

The ELF library gives us a set of functions to extract information about the file from the header. This allows us to understand information about the different sections in the file and tells us how we should organise these in the durvasa memory structure. The type of information that is read in by the ELF library is shown below.

| print.o: file format elf32-i386 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sections: |  |  |  |  |  |
| Idx Name | Size | VMA | LMA | File off | Align |
| 0 .init | 0000002c | 08048080 | 08048080 | 00000080 | 2**4 |
| 1 .text | 00011118 | 080480b0 | 080480b0 | 000000b0 | 2**4 |
| 2 .fini | 0000001c | 080591d0 | 080591d0 | 000111d0 | 2**4 |
| 3 .rodata | 00003c7d | 080591ec | 080591ec | 000111ec | 2**2 |
| 4 __libc_atexit | 00000004 | 0805ce6c | 0805ce6c | 00014e6c | 2**2 |
| 5 .data | 00000a90 | 0805de70 | 0805de70 | 00014e70 | 2**2 |
| 6 .ctors | 0000000c | 0805e900 | 0805e900 | 00015900 | 2**2 |
| 7 .dtors | 00000008 | 0805e90c | 0805e90c | 0001590c | 2**2 |
| 8 .got | 00000010 | 0805e914 | 0805e914 | 00015914 | 2**2 |
| 9 .bss | 00000868 | 0805e924 | 0805e924 | 00015924 | 2**2 |
| 10 .comment | 0000003c | 00000000 | 00000000 | 00015924 | 2**0 |
| 11 .note | 0000003c | 0000003c | 0000003c | 00015960 | 2**0 |

Figure 4.2, Information extracted by the ELF library

From the above table we can see that a basic 106 k program that prints ‘Hello World’ to the screen has 11 different sections. The .init, text and  .fini sections are all part of the text segment. The .rodata, __libc_atexit, .data, .ctors, .dtors, and .got sections are all part of the data segment. The .bss segment represents the bss segment, and the .comment and .note sections contain comments and other information such as the names of subroutines in the program.

The Size field is self explanatory, it tells us how long the section is in the file, and how long it will take in the memory when we load it into the memory.

The VMA (Virtual Memory Address) and LMA (Logical Memory Address), both specify the position that the specific section should be loaded into virtual memory, although physically it can be loaded anywhere in the memory as long as the addresses are mapped correctly.

The File Off field tells us how many bytes from the start of the file the specific section is located. We can see that the first section is located 0x80 bytes from the start of the file. This tells us that the first 0x80 bytes of the file is reserved for the header of the file.

The Align field specifies the alignment required in the specific section. The alignment tells us how many bits in the address should be zero at the start of the next section. E.G. for the first section in the above diagram the start address is 0x8048080 and the end address is 0x080480AC. The alignment required in this section is 4 bit alignment and we can conclude from this that the start of the next section will be the next address with the last 4 bits set to 0. The next address after 0x80480AC that has 4 bit alignment is 0x80480B0, which is made the start of the next section. The space between 0x80480AC and 0x80480B0 is filled with zeroes.

Alignment creates efficiency when we read from a block of memory. If we write to a piece of memory in a way that doesn’t align, when we read it again we may have to do more than one read from the memory to get the data. If it aligns then we can read that data with only one read.

Chapter 5

Program Structure

The main components of the current emulator are: Dsim_main, Dsim_core, Dsim_elfer, Dsim_processor, Dsim_syscall, Dsim_decode, Dsim_instruction, Dsim_error, Dsim_register.

The Dsim_main is the main entry point into the program. This creates a Dsim_core instance, and then goes on to create a Dsim_processor instance and starts the proceessor. The diagram below shows these components and how they interact with each other.

Dsim_main

Dsim_core

Creates / Starts

Uses

Creates / Starts

Dsim_elfer

Dsim_error

Uses

Uses

Dsim_syscall

Dsim_instruction

Dsim_Processor

Dsim_register

Dsim_decode

Uses

Uses

Uses

Dsim_register

Figure 5.1: Program Structure

The Dsim_core is the memory structure of the program. This represents a very basic process image of what would be present on the computer running the program. The Dsim_core holds an array which contains the four major sections that are present in the process view. These sections are shown in the diagram below,

Stack

Text Segment

Data Segment

BSS Segment

Figure 5.2 – The Sections Stored in the Core

These are the Stack, The Text segment, The Data segment, and The BSS segment. These are held in the array in the order mentioned above.

Data about these sections are held in a separate array that is used to read parts of the memory, but more importantly are used to map the virtual memory addresses to physical memory addresses. For example in the print and the add object file the text segment starts at the address 0x8048080. This is located in physical memory straight after the stack, at the physical address 0x10000. This means that every read from the memory requires a conversion from a virtual address to a physical address. The core has methods that take care of reading and writing to the memory, and also takes care of all the virtual address to physical address conversions required to read and write to the memory.

The Dsim_elfer is a helper class. Its function is to fill the core with data from the ELF file. It uses the libelf (The ELF library) functions to gain access to the data in the ELF based executable. It keeps the functionality with dealing with the file within itself. This is the only place in the simulator where the file is accessed. This makes it quite modular.

The Dsim_processor is the actual representation of the Pentium processor. This class contains a basic implementation of what a Pentium processor does. This class creates both the Dsim_decoder and Dsim_syscall classes, which carry out the decoding and system calls respectively. The Dsim_processor class holds all the main code for the execution phase of the emulation. The processor also creates instances of Dsim_register and Dsim_instruction as necessary. The Dsim_instruction instances are given to the Dsim_decoder and come back filled with the data needed to execute the instruction. The processor also needs to access the memory and thus contains a reference to the Dsim_core.

Two essential elements of the processor are the Stack Pointer and the Instruction Pointer. These are represented by two registers, these being the ESP (the Extended Stack Pointer) and the EIP (Extended Instruction Pointer).

On creation of the processor, the processor loops, taking each instruction and executing it. To get the instructions, an array with 16 bytes from the current EIP position is created and a pointer to this is given to the decoder. Also a pointer to a created Dsim_instruction instance is given to the decoder. The decoder fills the fields of the Dsim_instruction instance as necessary and the ‘executor’, which executes the specified instruction, uses this data.

The Dsim_register class handles the register data. An instance of this class is created for each register that is in the processor. Most of these registers contain 32 bits of information in them. It is necessary at times to access parts of these registers. For example, the EAX register is 32 bits but the AX register is nothing more than the lower 16 bits of the EAX register. There are functions in the Dsim_register class that use bit masking to give the correct number of bits required.

The Dsim_instruction class represents an assembly instruction. A new instance of this class is created for each new instruction that is executed. This class contains the necessary fields to determine the nature of the instruction when the processor executes it.

The Dsim_decoder class performs the decoding of the instructions. The decoder is given a pointer to an array with 16 bytes of data from the current EIP, and also a pointer to a Dsim_instruction instance that is created in the processor. It decodes the instruction and fills the Dsim_instruction instance with the required information for the decoder. It also returns an integer representing the number of bytes that have been taken from the text segment for the current instruction.

The Dsim_syscall class performs required system calls. As soon as the instruction int 0x80 is reached in the execution the Dsim_syscall is put into action. Currently only a few system calls have been implemented.

5.1 System Calls

System calls are routines that are carried out by the operating system to perform certain tasks that are commonly performed, e.g. printing data to the screen, or writing to a file. One of the biggest reasons for having system calls is security. What is meant by this is say we have a system in which each process is allowed to do its own thing when it comes to doing system calls, then what happens if more than one process tries to write to the disk at one time. This could cause corruption, and if someone writes a program that reads and writes to the disk differently than someone else’s program then other people’s data may be over written. System calls are carried out in the Pentium instruction set by starting an interrupt. More specifically, an int 0x80 instruction carries out a system call. The value in the EAX register specifies which system call is made. Some of the system calls and ways of implementation are outlined below.

NR_exit. Terminates the calling process immediately. Any open file descriptors belonging to the process are closed. In Durvasa this system call is implemented by terminating the current emulation.

NR_read. Attempts to read a specified number of bytes from file descriptor. This system call is currently not implemented in the emulator.

NR_write. Attempts to write a specified number of bytes to a file descriptor. This system call is currently not implemented in the emulator.

NR_open. Attempts to open a file and return a file descriptor. This system call is not implemented in the emulator.

NR_close. Closes a file descriptor, so that it no longer refers to any file and may be reused. . This system call is not implemented in the emulator.

NR_getpid. Returns the process ID of the current process. This system call is currently implemented by returning a value of 0. This is because for a simple emulation this system call is not important.

NR_getuid. Returns the real user ID of the current process. This system call is currently implemented by returning a value of 0. This is again because for a simple emulation this system call is not important.

NR_getgid. Returns the real group ID of the current process. This system call is currently implemented by returning a value of 0. This is again because for a simple emulation this system call is not important.

NR_getegid. Returns the effective group ID of the current process. This system call is currently implemented by returning a value of 0. This is again because for a simple emulation this system call is not important.

NR_acct. When called with the name of an existing file as argument, accounting is turned on, records for each terminating process are appended to filename as it terminates.  An argument of NULL causes accounting to be turned off. This system call is currently implemented by returning a value of 0. This is again because for a simple emulation this system call is not important.

NR_personality. Personality will make the execution domain referenced by persona (one of the parameters) the new execution domain of the current process. This system call is currently implemented by returning a value of 0. This is again because for a simple emulation this system call is not important.

Chapter 6

Results

Durvasa currently performs emulations on simple programs, currently being sucessful on a simple add program. The add program, shown below,  simply takes an int with initial value 0 and adds one to it.

int main()

{

int a = 0;

a = a + 1;

return 0;

}

The above program was compiled with cc in COREL LINUX, which produced a 106k object file. A snapshot of the emulation is shown below which shows the instructions that are performed in the main loop. We can see from the snapshot that this is where the ‘a = a + 1’ takes place.

0x80480e5 CALL 8048140

- Calls subroutine at 8048140. This calls the main in the code and pushes the return address onto the stack

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffec

EBP = 0 ESI = 0 EDI = 0

0x8048140 PUSH EBP

- Pushes the EBP register onto the stack

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffe8

EBP = 0 ESI = 0 EDI = 0

0x8048141 MOV ESP, EBP

- Moves the ESP register value to the EBP register

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffe8

EBP = ffe8 ESI = 0 EDI = 0

0x8048143 SUB 0x04, ESP

- Subtracts 4 from the ESP register

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffe4

EBP = ffe8 ESI = 0 EDI = 0

0x8048146 INC 0xfffffffc[EBP]

- Looks at the double word at address EBP – 4 (0xfffffffc is negative 4) and increases it. This is the ‘a = a + 1;’ part of the program

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffe4

EBP = ffe8 ESI = 0 EDI = 0

0x8048149 MOV EBP, ESP

- Moves the EBP register value back into the ESP register

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffe8

EBP = ffe8 ESI = 0 EDI = 0

0x804814b POP EBP

- Pops a double word off the stack and puts it into the EBP register

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = ffec

EBP = 0 ESI = 0 EDI = 0

0x804814c RET

- Returns from subroutine. This pops a double word off the stack, which will be the return address

EAX = 8059180 ECX = 0 EFLAG = 801

EDX = 0 EBX = 1 ESP = fff0

EBP = 0 ESI = 0 EDI = 0

An attempt was made to emulate a simple print program as shown below. An object file about the same size was obtained, with a much more complicated main method. The emulator reached the main method, but for some unknown reason never reached a system call to write to a stream. This attempt was very close to succeeding, and with more time could be completed.

int main()

{

printf(“Hello World\n”);

return 0;

}

Chapter 7

Problems

In the process of writing the emulator a number of problems have arisen from the initial implementation, and also from the current work.

The first thing that came up was the problem with the EIP. The previous implementation of the emulator stored a pointer in the EIP, which pointed to the memory buffer in the core. This was done for the reason that this worked with the previous implementation of the decoder. Previously the pointer was given to this decoder and it used this pointer to read the instructions from the core. This didn’t allow an easy implementation of instructions that change the program flow and replaced the EIP with immediate information. This was overcome by storing the virtual memory address in the EIP rather than a pointer. As a consequence of this a new way of giving the decoder the instruction data from the core had to be devised. This was overcome by separately getting 16 bytes of information from the current position in the core, storing this in an array, and then giving a pointer to this array to the decoder.

Another problem that was found was with the core. A call instruction was made to the very first section of the text segment. No data was found at this position, and a close look at the core showed that it was regarding the .text section in the file as being the first rather than the .init section. A simple change to a line of code in the core fixed this problem.

Later a second problem with the core was found. The data in the data segment was not correct. Later I noticed that the data was a couple of bytes away from where it was meant to be. The length of each section was not being calculated properly because the alignment was not being taken into account. The sections were being loaded correctly into the core, but the calculation to map the virtual address to the physical address was not being performed properly. This was because of the misrepresentation of the length of the sections.

A more general problem that I had was with the complexity of the task to create an emulator. The documentation that was used at times was not clear about specific things, and debugging the code was a very time consuming and complicated task.

Chapter 8

Conclusion

Pentium processors are the most popular processors around at the moment for many reasons, most notably their power, flexibility, and good value for money compared to their competitors. Intel’s attempt to keep backwards compatibility has complicated things somewhat. This has resulted in the Pentium processor having a large number of instructions. The effort to incorporate RISC elements into the Pentium has been complicated by the efforts to keep backward compatibility. This has meant that they cannot go completely with a RISC based system. The effort needed to write an emulator would have been greatly simplified in the case of Pentium’s being completely RISC based, and arguably the Pentium would have been faster, and more efficient if a RISC based solution was implemented.

The current emulator has extended the capabilities of the initial emulator in such a way as to allow emulation of simple programs. A new decoder has been written and an executor has also been added. A number of problems have been run into along the way, which has meant that the progress has been slowed. This emulator can be extended in the future to allow more complicated emulations and to also add simulation capabilities.

#### Apendix A

#### Source Code

#### The source code for the Durvasa project is not included in this report due to space restrictions. The source can be downloaded from the archive at the following Web site:

#### http://www.tcs.auckland.ac.nz/~btech/btech2000/pmcc042/source.tar.gz.

#### Bibliography

[Intel 99a]	Intel corp. Intel Architecture Software Developers Manual Volume 1:Basic Architecture , Intel Publishing 1999

[Intel 99b]	Intel corp. Intel Architecture Software Developers Manual Volume 2: Instruction Set Reference, Intel Publishing 1999

[ELF 99]	Portable Format Specifications, Version 1.1 Executable and Linking  Format (ELF), Total interface standards 1999

[Digital 00]	Digital Think Inc.– e-learning. www.digitalthink.com/. Pentium Processor Architecture. Digital think 1998.